### Detailed Shift Plan

#### High Level Goals:

- Establish Streaking in MRCO

- Study the resonant AM yield of a molecular ion and a fragment ion from the same core-

excited state.

  - Expect shifts in the molecular peak due to change in the geometry.

  - Sigma* resonance at the F edge. Does the excitation energy matter? No

  - CF4

    - Previously observed ultrafast decay.

    - Energy shift of 10 eV between early and late decay patterns.

    - Some interesting Doppler splittings

    - Needs the relative distribution of the TOFs to be well understood.

- Differentiate the early and late decay patterns in CF4, and derive the distribution in time.

- Prioritize on single shot reconstruction following angular streaking.

Goals discussed on 10/11/2024:

- Step 0: All TOFs tuned for Carbon 1s angular streaking

  - ~690 eV (at/around resonance) and 750 eV (above)

- Step 1: Reserve the TOFs for Doppler shift (ports 0, 180, 90, 270).

- Step 2: Reserve most number of TOFs among the rest of the 12 TOFs for Carbon Auger, followed by F Auger

- Keep SF6 as a backup. More discussion needs to happen about changed sample.

- Need to think about a backup plan for gas mixing with Kr with CF4.

- Relative efficiency of the TOFs needs to be understood.

#### Shift :

- Bring beam down to TMO (45 min)

  - Start with 400 eV photon energy.

  - Beam to IM5K4

  - Check KBO transmission

  - On the sample paddle:

    - Check x-ray size on the paddle with the microscope for:

      - Nominal focus, MRCO position (check spot size).

      - Push focus downstream:
set_KB1(bender_preset=’50umIP_z50mm’)

      - See the focus get larger on the microscope paddle.

  - CheckIP timing on sample paddle diode

    - Check x-ray signal

    - Time laser signal

- Timing ATM (1 hour):

  - Spatial Overlap laser on IMATM

  - Coarse timing on IMATM diode

  - Fine timing on ATM target (txt)

- Timing at IP with paddle (30 min):

  - Paddle scan on SiN target for fine timing.

- Setup XLEAP (~1 hours) w/o spectrometer (reload config).

- Setup FZP spectrometer at 400 eV.

Decision Point whether 690 or 400 eV

Should we go to 690 eV + associated FZP setup?

- Re-establish ATM timing with XLEAP.

- Use Argon to do angular streaking at 400 eV.

- Change sample to CF4

  - Be at photon energy of 400 eV and look at Carbon 1s photo.

- For 690 eV, do a photon energy scan from 680 to 710 eV using CF4.

  - Step 0: All TOFs tuned for Carbon 1s angular streaking

    - ~690 eV (at/around resonance) and 750 eV (above)

- Commission 690 eV Zone plate for Photon spectrometer. (1 hour)

- Photon energy scan of CF4 between 680 to 710 eV, single pulse.

- Laser Spatial/temporal Overlap:(90 min)

  - Check spatial overlap on sample paddle

  - Ask for more energy and check timing on the sample paddle

- Switch to XLEAP

  - Check IM2K4 and IM5K4: did the beam drift

  - Check ATM spatial overlap:

  - Check ATM signal (might be very weak due to less pulse energy)

  - Check spatial overlap on sample paddle

Shift 2:

High level goal.

  - 0, 90, 180, 270 (F edge - 630 V), the rest of the carbon (20 V).

  - w/2w setting XLEAP. (345/690 to 375/750)

  - Establishes ATM time signal.

  - Hf-2hf PV scan.

  - Absorption scan with SASE.

  - 750 eV above threshold. (Can we go there? Yes)...

    - F or C 1s for time t = 0

    - Find the FZP to go over there 750 eV

    - Make sure counts are as high as possible.

Plan:

  - Bring beam down to TMO (45 min)

  - Start with 400 eV photon energy.

  - Beam to IM5K4

  - Check KBO transmission

  - On the sample paddle:

    - Check x-ray size on the paddle with the microscope for:

      - Nominal focus, MRCO position (check spot size).

  - Timing ATM (1 hour):

    - Spatial Overlap laser on IMATM

    - Coarse timing on IMATM diode

    - Fine timing on ATM target (txt)

  - Timing at IP with paddle (30 min):

    - Paddle scan on SiN target for fine timing.

  - Absorption scan with SASE settings (between 665 - 705 eV).

  - Setup the TOF spectrometer in the interleaved settings:

    - 0, 90, 180, 270 (F edge - 630 V), the rest of the carbon (20 V).

    - hf_2hf PV scan.

  - 750 eV to excite above resonance of F.

Shift 3:

We see very little angular streaking.

  - Bring beam down to TMO (45 min)

  - Start with 400 eV photon energy.

  - Beam to IM5K4

  - Check KBO transmission

  - On the sample paddle:

    - Check x-ray size on the paddle with the microscope for:

      - Nominal focus, MRCO position (check spot size).

      - Move to Ming-Fu, 30 um focus spot size.

      - Do spatial overlap of IR/X-ray. Step at different focal positions in X and Y to find the range of movement.

  - Timing ATM (1 hour):

    - Spatial Overlap laser on IMATM

    - Coarse timing on IMATM diode

    - Fine timing on ATM target (txt)